# Lab Report Number: 04

# Wide Area Networks

| Student Number | Student name: | Contribution Percentage: |
| --- | --- | --- |
| s4091918 | Caitlin Frank | 50% |
| s4181317 | Henrietta Barry | 50% |
| Group: |  |  |
| Session: | 3 – Tuesday 12:30 lab |  |
| Lab Demonstrator Name: | Charith Dissanayake |  |

# ABSTRACT

Place your abstract here. The abstract should answer the following questions:

• What is the general objective of the lab?

• What type of work is performed to achieve the objective?

# OBJECTIVE AND INTRODUCTION

Place your sections objectives and introduction here. State the objectives clearly in complete sentences. A few explanatory sentences may be included, if needed.

ABSTRACT	1

OBJECTIVE AND INTRODUCTION	1

RESULTS AND DISCUSSION	1

Q1	2

Q2	2

Q3	2

PART1	2

Q4	2

Q5	3

Q6	3

Part 2	6

Q8	6

Q9	6

Part 3	8

Q10	8

Q11	10

Q12	10

Q13	10

Part 4	10

Q14	10

Q15	10

Q16	10

CONCLUSIONS	10

# RESULTS AND DISCUSSION

### Q1

Figure 1: from lab 4 guideline

Parts of the WAN component/OSI Layer

Routers – Routers connect your local area network (LAN) to the wide area network (WAN). Within a LAN network, routers speed up communication between devices. However, their function as a WAN component is to determine best path for transmission and handles network address translation (NAT) from LAN to WAN networks as well as providing security.

Switches – Switches connect devices within a LAN network.

### Q2

### Q3

## PART1

### Q4

PC1.1 config

PC1.2 config

PC1.3 config

PC2.1 config

PC2.2 config

### Q5

### Q6

Test LAN by pinging PCs

NETWORK 1

PC1.1 pinging PC1.2

PC1.2 pinging PC1.3

NETWORK 2

PC2.1 pinging PC2.2

PC2.2 pinging PC2.1

Q7.

Because PC2.1 and PC2.2 are on the same subnet there is only 1 tracing route because the packet is transferred directly though the switch, not through a router or otherwise. There is only the IP of the destination, this is because while the whole path is usually displayed in this case there is only one device in this path (the destination). The switch does not appear on the traceroute because switches operate in layer 2 (data link layer), not layer 3 (network layer).

## Part 2

Edge Router 1 & 2 configuration

### Q8

This lab uses /24 subnet mask. This is because it is a simple easy to remember address (255.255.255.0), no complicated subnet calculations required to convert /24 to binary form. This means it is often used for beginners to networking. It also means that there are spare usable addresses in the network so that you have to learn to configure the correct link. Alternatively using /30 (255.255.255.252) means that there are 2 IP addresses (one for each router interface). This means it is best for point to point router links.

### Q9

Pinging PC2.1 from PC1.1 to test connectivity between network 1 and 2. Including the trace route using tracert command in PC1.1 command prompt. (below)

Pinging PC2.2 from PC1.2 to retest connectivity between network 1 and 2. Including the trace route using tracert command in PC1.2 command prompt.(below)

## Part 3

TTL (time to live), used to control the amount of time a packet is sent through routers to ovoid the packet being sent indefinitely. Packets include a numerical value which determines the TTL value, which is subtracted from by each router. When a TTL value = 0 the router will send an ICMP message back to the command prompt window.

### Q10

Traceroute from PC4.1 to PC2.1. To check the connectivity between each router and switch in the route I individually pinged each hop manually.

The tracert function matches the hops I took when manually pinging each point in the path manually. The first number indicates the hop number (TTL value), the 3 millisecond values indicate the time per round trip. Then the IP address of the host router. The first-round trip at hop 4, showing a sudden increase from 1-2 to 11ms indicates a jitter in that packet transfer.  Otherwise, the connection is quite steady. (switches are not shown in tracrt because they operate in layer 2 not layer 3)

### Q11

The tracert function indicates there are 2 hops on this route. Hop 1 is the default gateway from network 1, packet is forwarded to this address (192.168.1.1). Hop 2 is the destination PC itself, the packet reached its final destination and responded. Router 2 does not appear on the tracert because the TTL value did not equal 0 at this location, therefore not responding with a ICMP message.

### Q12

Path from PC4.1 - PC2.1 = PC4.1 -> router4 -> router1 -> router2 -> switch2 -> PC2.1

Path from PC1.1 - PC2.1 = PC1.1 -> router1 -> router2 -> switch2 -> PC2.1

Common channels:

- Router1 -> Router2

- Router2 -> Switch 2

- Switch2 -> PC2.1

Edge router 1 is the convergence point for traffic from network 4 and network 1.

We can see that the average packet transfer time for the connection between PC1.1 and PC2.1 is faster than the connection between PC4.1 and PC2.1, 1ms and 2ms respectively. This is because the first path is a shorter one requiring transfer between less routers.

### Q13

## Part 4

### Q14

### Q15

### Q16

CONCLUSIONS

Place your conclusions here. State your discoveries, judgments and opinions from the results of this experiment.

# REFERENCES